# Sopers Road Car Park Resurfacing and Relining SOW

Northaw & Cuffley Parish Council, Version: Draft v1.0

Statement of Works (SOW) and Annex A Technical Specification

Project: Sopers Road Car Park Resurfacing and Relining
Client: Northaw & Cuffley Parish Council
Location: Sopers Road Car Park, Cuffley

## Purpose

## The purpose of this Statement of Works (SOW) is to define the scope, objectives, deliverables, and standards for the resurfacing and relining of Sopers Road Car Park in Cuffley. The works aim to restore the surface condition, improve safety and accessibility, and ensure compliance with the Disability Discrimination Act (DDA) and relevant British Standards.

A1. General Requirements

- The Contractor shall provide all labour, plant, materials, and supervision necessary to complete the works described in the Statement of Works (SOW).

- All materials and workmanship must conform to current British Standards, Highways Agency Specifications (Series 900), and manufacturer’s recommendations.

- The Contractor must visit the site to assess the full extent of the works prior to commencement and confirm all quantities and requirements.

- All waste material, arisings, and debris shall be removed from site and disposed of in compliance with the Environmental Protection Act 1990 and Duty of Care Regulations.

A2. Site Preparation and Excavation

- Remove the existing asphalt surface using planing equipment or excavation to an average depth of 50mm–80mm, depending on surface condition.

- Excavate any failed or unstable areas to a depth sufficient to expose solid sub-base.

- Inspect the existing sub-base; any soft spots shall be excavated and replaced with Type 1 granular sub-base material (to Clause 803 of the Specification for Highway Works) compacted to refusal.

- Sweep and clean all surfaces prior to laying new materials.

A3. Surfacing Specification

A3.1 Binder Course (if required)

- Material: AC 20 Dense Binder Course 100/150 PSV40 or equivalent.

- Thickness: 40mm nominal compacted depth.

- Compaction: Roll with twin-drum or vibratory roller to achieve uniform finish.

A3.2 Surface Course

- Material: AC 10 Close Surface Course 100/150 PSV55 (bituminous asphalt to BS EN 13108-1).

- Thickness: 40mm nominal compacted depth.

- Binder: 100/150 pen bitumen or equivalent approved material.

- Finish: Smooth, even, and free of depressions or standing water.

- All joints shall be treated with bitumen edge sealant and rolled to ensure proper bonding.

A3.3 Edging and Drainage

- Reinstate or repair all kerb edges, drainage channels, and manhole surrounds as necessary.

- Ensure surface gradients fall correctly to existing gullies to prevent ponding.

- Replace damaged gully covers or frames where required.

A4. Line Marking

- All markings to be laid after the surface has fully cured (minimum 24 hours after completion).

- Material: Hot-applied thermoplastic or high-performance cold-applied line marking compound, complying with BS EN 1871 and BS EN 1436.

- Colours:

  - White – standard bay lines and directional arrows.

  - Yellow – disabled parking bays and restricted areas.

- Line width: 100mm standard; 150mm for disabled bays and pedestrian routes.

- Reapply all existing markings including:

  - Standard bays (approx. 83).

  - Disabled bays (minimum 2).

  - Directional arrows.

  - Hatched areas and pedestrian walkways.

  - Any stop or give-way lines as per the existing layout.

A5. Accessibility and DDA Compliance

- Ensure disabled parking bays meet BS 8300 and Equality Act 2010 requirements.

- Minimum disabled bay dimensions: 3.6m width × 6.0m length (including hatched transfer area).

- Install appropriate signage or ground markings with wheelchair symbols.

- Ensure pedestrian access routes are clearly delineated and level, with tactile indicators where appropriate.

A6. Traffic and Site Management

- The Contractor must submit a Traffic Management Plan prior to starting works (if required) car park can be closed.

- Works must be carried out in a manner that ensures the car park remains safely accessible or clearly closed off.

- Use barriers, cones, and warning signs to control vehicle and pedestrian movement.

- Work hours and phasing must be agreed in advance with the Council to minimise disruption.

A7. Quality Control and Testing

- All materials shall be accompanied by delivery tickets and compliance certificates.

- Compaction and surface testing to meet BS 594987 standards.

- The finished surface must be inspected by the Council’s representative prior to acceptance.

- Any defects or failures within 12 months of completion must be made good at the Contractor’s expense.

A8. Completion and Handover

- Upon completion, the Contractor shall:

  - Remove all plant, materials, and waste from site.

  - Sweep and clean the entire area.

  - Provide a Completion Certificate confirming compliance with specification.

  - Provide photographic records before and after works.

  - Submit warranties for materials (minimum 12 months).

A9. Programme and Timescales

- Anticipated duration: one week, subject to weather conditions.

- Working hours: 8:00 a.m. to 5:00 p.m., Monday to Friday, unless otherwise agreed.

- Contractor to provide a detailed programme of works including mobilisation, surfacing, curing, and line marking phases.

A10. Deliverables

- Fully resurfaced and relined car park meeting all specification and DDA standards.

- Supporting documentation:

  - Risk assessments and method statements (RAMS).

  - Waste disposal records.

  - Material compliance certificates.

  - As-built plan (if layout amended).

  - Completion and warranty certificates.

## Annex B – Pricing Schedule Template

## Project: Sopers Road Car Park Resurfacing and Relining
Client: Northaw & Cuffley Parish Council
Location: Sopers Road Car Park, Cuffley

## Instructions to Tenderers

## Tenderers must complete all relevant sections of this pricing schedule.

## Prices must be stated in GBP (£) and exclusive of VAT.

## All prices must be fully inclusive of labour, materials, plant, transport, waste disposal, supervision, and overheads.

## The Council will not accept any additional charges unless pre-approved in writing.

## Tenderers should clearly identify any optional or provisional items.

## The completed pricing schedule will form part of the formal contract upon award.

## A. Summary of Works

| ## Ref | ## Description of Work | ## Unit | ## Quantity (Est.) | ## Unit Rate (£) | ## Total Cost (£) |
| --- | --- | --- | --- | --- | --- |
| ## 1 | ## Site establishment, signage, and traffic management | ## Item | ## 1 |  |  |
| ## 2 | ## Plane off and remove existing asphalt surfacing (average 50–80mm depth) | ## m² |  |  |  |
| ## 3 | ## Supply and lay AC 20 Dense Binder Course 100/150 PSV40 (if required) | ## m² |  |  |  |
| ## 4 | ## Supply and lay AC 10 Close Surface Course 100/150 PSV55 (40mm compacted) | ## m² |  |  |  |
| ## 5 | ## Reinstatement/repair of kerbs, drainage channels, and manhole surrounds | ## Item |  |  |  |
| ## 6 | ## Supply and apply thermoplastic or cold-applied line markings (white/yellow as specified) | ## m |  |  |  |
| ## 7 | ## Re-marking of disabled bays (3.6m × 6.0m incl. hatching and symbols) | ## No. |  |  |  |
| ## 8 | ## Re-marking of pedestrian walkways and directional arrows | ## Item |  |  |  |
| ## 9 | ## Environmental waste removal and lawful disposal | ## Item | ## 1 |  |  |
| ## 10 | ## Site cleaning and demobilisation | ## Item | ## 1 |  |  |

## Subtotal £__________
Contingency / Provisional Sums (if any) £__________
Total Tender Price (excl. VAT) £__________
VAT @ 20% £__________
Grand Total (incl. VAT) £__________

## B. Optional / Additional Works (if applicable)

| ## Ref | ## Description | ## Unit | ## Rate (£) | ## Comments |
| --- | --- | --- | --- | --- |
| ## A | ## Replacement of damaged drainage gully cover | ## Item |  |  |
| ## B | ## Replacement of kerb edging (per linear metre) | ## m |  |  |
| ## C | ## Supply and install new directional signage | ## Item |  |  |

## C. Programme and Delivery

| ## Item | ## Description | ## Contractor Response |
| --- | --- | --- |
| ## 1 | ## Anticipated project start date |  |
| ## 2 | ## Estimated duration of works (days/weeks) |  |
| ## 3 | ## Working hours and days |  |
| ## 4 | ## Lead-in time from contract award |  |

## D. Declarations

## The undersigned confirms that:

## All prices submitted are fixed and firm for the duration of the works.

## All costs necessary to complete the works to the specification have been included.

## The contractor has inspected the site and is satisfied with all conditions.

## The submission complies with the Parish Council’s tender requirements.

## Company Name: ____________________________________
Authorised Signatory: _______________________________
Position: __________________________________________
Date: _____________________________________________
Contact Email: _____________________________________
Telephone: ________________________________________